Resin8 Disposable Label — Printer Setup Guide

Cannavative Extracts, LLC  |  Zebra ZD420  |  50mm × 40mm Labels

SECTION 1 — Label Stock to Purchase

Label Specifications

- Size: 2" × 1.57"  (50mm × 40mm)  — or nearest standard: 2" × 1.5"

- Type: Direct Thermal (DT) — no ribbon required

- Color: White matte

- Adhesive: Permanent

- Core: 1" (fits ZD420 standard spindle)

- Winding: Outside wound

- Gap: 3mm gap/notch between labels

Recommended Sources

Option A — Zebra Genuine (Most Reliable)

- Part #: 10010060

- Description: Zebra Z-Select 2000D, 2" × 1.5" Direct Thermal

- Order from: zebra.com or any authorized reseller

- Approx. cost: ~$40–$60 per roll (800 labels/roll)

NOTE: Closest standard to 50×40mm. Label will print at 50×38mm — adjust ^LL to 305 if needed.

Option B — Uline (Budget, Ships Fast)

- Part #: S-19164  (2" × 1.5" thermal, 1" core)

- Order from: uline.com

- Approx. cost: ~$30–$40 per roll

Option C — Custom Cut (Exact 50×40mm)

- Vendor: Lightning Labels (lightninglabels.com) or StickerGiant

- Specify: 50mm × 40mm, direct thermal, 1" core, outside wound, 3mm gap

- Minimum order: typically 250–500 labels

NOTE: Best for bulk orders. Exact size eliminates any margin adjustment.

SECTION 2 — Zebra ZD420 Printer Setup

Step 1 — Install Label Stock

- Open the top cover by pressing the release button on the front.

- Pull the media guides apart and load the label roll so it unwinds from the BOTTOM.

- Feed the labels through the media slot until they extend past the front edge.

- Slide the media guides in until they lightly touch the edges of the label roll.

- Close the top cover.

Step 2 — Calibrate the Printer

- With labels loaded and printer powered on, hold the FEED button for 2 seconds.

- The printer will advance several labels and then stop — this calibrates the gap sensor.

- Print a configuration label to confirm: hold both FEED + CANCEL for 2 seconds.

Step 3 — Send One-Time Configuration

Connect the ZD420 to your computer via USB. Open Notepad, paste the following ZPL configuration block, save as "zd420-config.zpl", then send it to the printer using the command in Step 4.

^XA
^MMT              ; Tear-off mode
^PW399            ; Print width = 50mm (399 dots @ 203 DPI)
^LL319            ; Label length = 40mm (319 dots @ 203 DPI)
^MAN              ; Auto-sense label gap
^PR4,4            ; Print speed = 4 ips
^MD10             ; Darkness = 10 (adjust 0–30 if needed)
^LH0,0            ; Label home = top-left, no offset
^JMA              ; Save all settings to printer memory
^XZ

Step 4 — Send ZPL to Printer (Windows)

Open Command Prompt and run:

copy /b "zd420-config.zpl" \\\\.\\ USB001

NOTE: If your Zebra is connected to a different USB port, replace USB001 with USB002, etc. Check Device Manager > Ports if unsure.

Step 5 — Print a Label

- Open the label file: EXAMPLE-apple-jack.zpl

- Send to printer: copy /b "EXAMPLE-apple-jack.zpl" \\.\USB001

- The label should print immediately.

Darkness / Quality Adjustment

If labels print too light or too dark, resend the config with a different ^MD value:

- ^MD8  → lighter (use if ink is bleeding or smearing)

- ^MD10 → default starting point

- ^MD15 → darker (use if text appears faint)

- ^MD20 → very dark (use on rough or matte stocks)

SECTION 3 — Daily Printing Workflow

For each new batch:

- Open resin8-disposable.zpl in any text editor (Notepad works).

- Replace each [PLACEHOLDER] with the batch-specific value from your COA.

- Save the file with a new name (e.g., "CJ Apple Jack 125289.zpl").

- Send to printer: copy /b "CJ Apple Jack 125289.zpl" \\.\USB001

- Verify first label before printing the full run.

Placeholders Reference

- [STRAIN]         — Strain name only, e.g., Apple Jack

- [MM/DD/YY] ×3    — Produced, Tested, Packaged dates

- [PRODUCTION RUN NUMBER] — Full run number from METRC/COA

- [THC%] [THC mg]  — From COA, e.g., 87.89 and 878.9

- [CBD%] [CBD mg]  — From COA

- [CBC/CBG/CBN/THCV %] — Minor cannabinoids from COA

- [Terpene 1/2/3]  — Top 3 terpenes with percentages

SECTION 4 — Compliance Reference

All fields on this label are required by Nevada Cannabis Compliance Regulation 12, Section 12.035 (Production Facility label before sale to retail store).

Regulation source: https://ccb.nv.gov/wp-content/uploads/2025/07/Reg-12.pdf

Required Fields (NCCR 12.035)

- Establishment name + cannabis establishment ID  [§12.035(a)]

- Production run number  [§12.035(c)]

- "Keep out of reach of children"  [§12.035(d)]

- Date of production / testing / packaging  [§12.035(e)]

- Cannabinoid profile and potency levels  [§12.035(f)]

- Total THC in milligrams  [§12.035(h)]

- Net weight  [§12.035(k)]

- Extraction process and solvent used  [§12.035(l)]

- "THIS PRODUCT CONTAINS CANNABIS"  [§12.035(n)]

Font Requirements  [§12.010(3)]

- Minimum 8-point font — all text on this label uses ≥8pt

- No italic text permitted anywhere on the label

Not Required for Non-Edible Concentrates/Vapes

- Nevada universal cannabis symbol (required for edibles only — §12.015, §12.020)

- "This product may be unlawful outside Nevada" (sales facility obligation — §12.045)

- "This product may have intoxicating effects" (sales facility obligation — §12.045)